Lime Microsystems Limited

Surrey Tech Centre

Occam Road

The Surrey Research Park

Guildford, Surrey GU2 7YG

United Kingdom

Tel:		+44 (0) 1483 685 063

Fax:		+44 (0) 1428 656 662

e-mail:		enquiries@limemicro.com

Stream protocol

| Document version: | 01 |
| --- | --- |
| Document revision: | 00 |
| Last modified: | 12/07/2017 14:51:00 |

Contents

- Packet structure	4

1.1	Streaming configuration	5

1.1.1	12 bit compressed samples	7

1.1.2	16 bit compressed samples	7

Revision History

Version 01r00

Released: 23 Nov, 2015

Initial version.

Version 02r00

Released: 13 Jan, 2016

Updated FPGA registers map. Fixed bits indexes in samples data structures.

Version 03r00

Released: 28 Jan, 2016

Changed packet header byte indexes.

Version 04r00

Released: 17 Feb, 2016

Added timestamp reseting, timestamp synchronization individually for each packet.

Version 05r00

Released: 9 May, 2016

Updated FPGA registers map.

Version 06r00

Released: 12 July, 2017

Updated FPGA registers map for LimeSRD-USB 1v4 board.

Packet structure

Total packet size is 4096 bytes. It consists of two main parts: header and payload as described in Table 1. Header contains receiver and transmitter status flags and packet timestamp. Timestamp is a 64 bit samples counter used to synchronize received and transmitted signals. The counter is being incremented with each sample after Receiver is enabled. Timestamp can be reset to 0, by using SMPL_NR_CLR bit, packets streaming should be disabled when reseting timestamp. Payload contains RF samples data, the data format and ordering depends on number of active channels and each sample bit count.

Table 1 Packet structure

| Byte index | Bits | Description |
| --- | --- | --- |
| Header |  |  |
| 0 | 15-2 | Reserved |
|  | 4 | Disable timestamp synchronization for this packet. Gets OR’ed with SPI SYNCH_DIS 0 – synchronize packet transmitting with timestamp (For transmitting only) 1– ignore timestamp, transmit as soon as possible. Prior synchronized packets existing in FIFO can delay transmitting of unsynchronized packet. |
|  | 3 | Tx packet dropped: 0 - Tx is working normally 1 - Tx received packet with obsolete timestamp |
|  | 2-0 | FPGA Rx FIFO fill status: 0– from 0% to 12.5% 1 – from 12.5% to 25% 2 – from 25% to 37.5% 3 – from 37.5% to 50% 4 – from 50% to 62.5% 5 – from 62.5% to 75% 6 – from 75% to 87.5% 7 – from 87.5% to 100% |
| 1-7 | 15-0 | Reserved |
| 8-15 | 15-0 | Timestamp: 64 bit samples counter, stored in Big endian format When Receiving: timestamp when the first sample in payload was received When Transmitting: timestamp when the first sample in payload should be transmitted |
| Payload |  |  |
| 16-4095 | 15-0 | RF samples data: Samples format and ordering depends on streaming configuration |

  - Streaming configuration

Streaming configuration should be set before initiating data streaming. Protocol configuration is set by writing to board SPI registers. DNU - Do not use (register bit unsuported in device)

Table 2 Configuration registers

| Address | Def. value | Bits | Name | Description | LimeSRD-USB 1v4 |
| --- | --- | --- | --- | --- | --- |
| 0x0000 |  | 15-0 | Board ID |  | 000E |
| 0x0001 |  | 15-0 | GW version |  |  |
| 0x0002 |  | 15-0 | GW revision |  |  |
| 0x0003 |  | 15-7 | Reserved |  |  |
|  |  | 6-4 | BOM_VER |  |  |
|  |  | 3-0 | HW_VER |  |  |
| 0x0004 | 0000 | 15-0 | Reserved |  |  |
| 0x0005 | 0000 | 15-0 | DRCT_CLK_EN | Each bit enables clock to be direct clock source: 1 - enabled, 0 - disabled |  |
|  |  |  |  | [n] - nth clock |  |
|  |  |  |  | [1] - Second clock | RX clk |
|  |  |  |  | [0] - First clock | TX clk |
| 0x0006 | 0000 | 15-0 | Reserved |  |  |
| 0x0007 | 0003 | 15-0 | CH_EN | MIMO Channel enables each bit means: 1- enabled, 0-disabled |  |
|  |  |  |  | [n] - nth channel |  |
|  |  |  |  | [1] - channel 1 |  |
|  |  |  |  | [0] - channel 0 |  |
| 0x0008 | 0102 | 15-11 | Reserved |  |  |
|  |  | 10 | DLB_EN |  | DNU |
|  |  | 9 | SYNCH_DIS | Packets synchronization using timestamps: |  |
|  |  |  |  | 0 - Enabled |  |
|  |  |  |  | 1 - Disabled |  |
|  |  | 8 | MIMO_INT_EN | MIMO mode: |  |
|  |  |  |  | 0 - Disabled |  |
|  |  |  |  | 1 - Enabled |  |
|  |  | 7 | TRIQ_PULSE | TRXIQ_pulse mode: | DNU |
|  |  |  |  | 0 - OFF |  |
|  |  |  |  | 1 - ON |  |
|  |  | 6 | DDR_EN | DIQ interface mode: | DNU |
|  |  |  |  | 0 - SDR |  |
|  |  |  |  | 1 - DDR |  |
|  |  | 5 | MODE | Limelight port mode: | DNU |
|  |  |  |  | 0 - TRXIQ |  |
|  |  |  |  | 1 - JESD207 |  |
|  |  | 4-2 | Reserved |  |  |
|  |  | 1-0 | SMPL_WIDTH | Interface sample width selection: |  |
|  |  |  |  | "10" - 12bit, |  |
|  |  |  |  | "01" - 14bit, |  |
|  |  |  |  | "00" - 16bit |  |
| 0x0009 | 0003 | 15-2 | Reserved |  |  |
|  |  | 1 | TXPCT_LOSS_CLR | TX packets dropping flag clear: |  |
|  |  |  |  | 0 - Normal operation |  |
|  |  |  |  | 1 - Rising edge clears flag |  |
|  |  | 0 | SMPL_NR_CLR | Reset_timestamp: Rx and Tx should be stopped when resetting |  |
|  |  |  |  | 1 - Timestamp is cleared |  |
|  |  |  |  | 0 - Normal operation |  |
| 0x000A | 0000 | 15-10 | Reserved |  |  |
|  |  | 9 | TX_PTRN_EN | Test pattern on TX: |  |
|  |  |  |  | 0 - Disabled |  |
|  |  |  |  | 1 - Enabled |  |
|  |  | 8 | RX_PTRN_EN | Test pattern on RX: | DNU |
|  |  |  |  | 0 - Disabled |  |
|  |  |  |  | 1 - Enabled |  |
|  |  | 7-2 | Reserved |  |  |
|  |  | 1 | TX_EN | TX chain: |  |
|  |  |  |  | 0 - Disabled |  |
|  |  |  |  | 1 - Enabled |  |
|  |  | 0 | RX_EN | RX chain: |  |
|  |  |  |  | 0 - Disabled |  |
|  |  |  |  | 1 - Enabled |  |

    - 12 bit compressed samples

When using 12 bit compressed samples configuration the packet payload has the following structure. Bytes are indexed from payload start.

Table 3 Samples data structure

| Byte index | Bits | Description |
| --- | --- | --- |
| 0 | 7-0 | ch0_I0 [7:0] |
| 1 | 7-4 3-0 | ch0_Q0[3:0] ch0_I0 [11:8] |
| 2 | 7-0 | ch0_Q0[11:4] |
| 3 | 7-0 | ch1_I1 [7:0] |
| 4 | 7-4 3-0 | ch1_Q1[3:0] ch1_I1 [11:8] |
| 5 | 7-0 | ch1_Q1[11:4] |
| … | … | … |

    - 16 bit compressed samples

When using 16 bit compressed samples configuration the packet payload has the following structure. Bytes are indexed from payload start.

Table 4 Samples data structure

| Byte index | Bits | Description |
| --- | --- | --- |
| 0 | 7-0 | ch0_I0 [7:0] |
| 1 | 7-0 | ch0_I0 [15:8] |
| 2 | 7-0 | ch0_Q0[7:0] |
| 3 | 7-0 | ch0_Q0[15:0] |
| 4 | 7-0 | ch1_I1[7:0] |
| 5 | 7-0 | ch1_11[15:8] |
| 6 | 7-0 | ch1_Q1[7:0] |
| 7 | 7-0 | ch1_Q1[15:8] |
| … | … | … |